EXPLORATORY DATA ANALYSIS

French Day-Ahead Power Market  |  2018–2024

EDHEC MSc Data Analysis & AI  —  Master’s Thesis

# 1. Introduction

This report summarises the exploratory data analysis (EDA) conducted on the French day-ahead electricity market. The dataset covers hourly observations from January 2018 to December 2024 (61,344 hours), combining four sources:

- ENTSO-E Transparency Platform: day-ahead prices, load forecast, generation by fuel type

- ERA5 (Copernicus / ECMWF): hourly 2m temperature, 10m wind speed, solar radiation, precipitation

- Derived features: Heating Degree Days (HDD), wind power proxy, Weather Stress Index (WSI)

# 2. Price Dynamics

## 2.1 Descriptive Statistics

The table below summarises the distribution of French day-ahead prices over the full sample period.

| Statistic | Value |
| --- | --- |
| Mean | 94.50 €/MWh |
| Median | 57.88 €/MWh |
| Std. Deviation | 104.34 €/MWh |
| Minimum | −134.94 €/MWh |
| Maximum | 2,987.78 €/MWh |
| Negative-price hours | ~2.1% of observations |
| Hours above 200 €/MWh | ~8.4% of observations |

The large gap between mean (94.50) and median (57.88) reflects strong positive skewness driven by crisis episodes in 2021–2022 (gas supply shock, post-COVID demand rebound, nuclear maintenance wave). Negative prices occur mainly during low-demand weekends with high solar and wind output.

Figure 1 — French day-ahead prices 2018–2024 (hourly). The 2021–2022 energy crisis is clearly visible.

Figure 2 — Price distribution (left) and annual boxplots (right). The 2022 outliers compress the scale.

## 2.2 Seasonality

Prices exhibit strong intra-day, intra-week, and seasonal patterns. The heatmap (Figure 4) is particularly informative: morning peaks (7–9h) and evening peaks (18–20h) are consistent year-round, while winter months show structurally higher prices due to heating demand.

Figure 3 — Average price by month, day of week, and hour of day.

Figure 4 — Heatmap of average price by hour × month. Dark red = high price, green = low price.

## 2.3 Nuclear Generation

France is structurally dependent on nuclear power (~70% of production). The 2022 crisis was amplified by an unprecedented nuclear maintenance wave that reduced availability to historical lows, pushing prices to extreme levels.

Figure 5 — Monthly average price (blue) vs. nuclear output (GW, orange). The inverse relationship is clear in 2022.

# 3. Weather Variables & Impact on Prices

## 3.1 Overview

ERA5 reanalysis data provides hourly weather observations spatially averaged over mainland France. Figure 6 shows the monthly evolution of the four weather variables used in the model.

Figure 6 — Monthly averages of ERA5 weather variables for France (2018–2024).

## 3.2 Temperature & Heating Demand

Cold temperatures drive up electricity demand through electric heating, a dominant effect in France. The U-shaped relationship in Figure 7 confirms this: both very cold and moderately warm periods see higher prices (cooling demand in summer), with the lowest prices in mild spring/autumn months.

Figure 7 — Scatter and binned average price by temperature. U-shaped pattern reflects dual heating/cooling demand.

Figure 8 — Monthly price vs. Heating Degree Days (HDD). High HDD periods (winter) correlate with higher prices.

## 3.3 Wind Speed & Renewable Generation

Wind generation in France (primarily onshore) can displace gas-fired peakers and lower prices significantly. Low-wind periods combined with cold temperatures create the most extreme price spikes.

Figure 9 — Price vs. wind speed. Higher wind speed is associated with lower prices (merit-order effect).

## 3.4 Weather Stress Index

The Weather Stress Index (WSI) is a composite variable defined as HDD divided by wind speed. It captures simultaneous high-demand and low-renewable-supply conditions — the configuration most likely to produce extreme price spikes. The correlation between WSI and price is 0.42 (Pearson), the highest of all individual weather features.

Figure 10 — Monthly WSI (orange dashed) vs. price (blue). Strong co-movement, particularly in winter 2021–2022.

## 3.5 Correlation Structure

The correlation matrix below confirms the expected relationships: nuclear output is negatively correlated with price, wind is negatively correlated, load positively correlated, and the WSI positively correlated.

Figure 11 — Correlation matrix across price, weather, and fundamental variables.

# 4. Key Findings & Modelling Implications

- Strong seasonality at hourly, daily, and monthly levels — calendar features (hour, day-of-week, month) are essential inputs.

- Price lags (T−24h, T−48h, T−168h) capture auto-regressive structure and regime persistence.

- Nuclear availability is the dominant French market fundamental — it should be treated as a primary feature.

- The Weather Stress Index (WSI = HDD / wind speed) has higher correlation with price than raw temperature or wind alone — justifying its inclusion as an engineered feature.

- Extreme price events (>200 €/MWh) cluster in 2021–2022; robust models should be evaluated with and without this crisis period.

Leo Camberleng — EDHEC MSc Data Analysis & AI — May 2026